# Aircraft Monitoring System

## Dual Aircraft Flight Control and Ground Station System

CST8227 - Interfacing
Final Project

Computer Engineering Technology - Computing Science
School of Advanced Technology

Submitted by:

Tatiana Fonseca (Student ID: 040934360)
Tisha Patel (Student ID: 041133762)

Submission Date: December 05, 2025

# Table of Contents

1. Abstract

2. Introduction

3. System Architecture

4. Pin Mapping Tables

5. Component List

6. Code Summary

7. Testing and Results

8. Conclusion

9. Fritzing

Appendix: Source Code

# 1. Abstract

This project implements a comprehensive dual-microcontroller system simulating aircraft flight control and ground station operations. MEGA-1 serves as the flight computer, managing altitude monitoring, landing gear deployment, attitude tracking via MPU6050, and motor control. MEGA-2 operates as the ground control station, handling environmental monitoring, RFID-based aircraft conflict detection, gate control, and radar simulation using a stepper motor. The system features secure passcode authentication, I2C inter-controller communication, rotary encoder-based airspeed adjustment during conflicts, and automated night mode operation.

# 2. Introduction

This project demonstrates embedded systems integration using two Arduino MEGA 2560 microcontrollers. The system simulates critical aviation operations including flight control, ground station monitoring, and conflict resolution. Key features include multi-sensor integration (ultrasonic, DHT11, MPU6050, RFID RC522), motor control (DC motors via L293D, servo motors, stepper motor), and real-time communication between microcontrollers.

# 3. System Architecture

## 3.1 MEGA-1: Flight Computer

MEGA-1 manages all flight-critical operations:

- Passcode authentication system with 4-digit entry

- Ultrasonic altitude monitoring with landing gear automation

- MPU6050 6-axis IMU for pitch, roll, and yaw tracking

- L293D motor driver for propeller/fan control

- Servo motor for landing gear deployment

- Active and passive buzzers for audio alerts

- 4-digit 7-segment display for passcode confirmation

- IR remote control for system reset

- I2C master communication to MEGA-2

## 3.2 MEGA-2: Ground Control Station

MEGA-2 manages ground station operations:

- DHT11 environmental monitoring (temperature/humidity)

- PIR motion detection with LED indicators

- RFID RC522 for aircraft conflict detection

- Ultrasonic sensor for object detection

- Servo motor for gate control

- L293D motor driver for cooling fan

- ULN2003 stepper motor driver for radar simulation

- Rotary encoder for airspeed adjustment

- Photoresistor for night mode detection

- I2C slave communication from MEGA-1

# 4. Pin Mapping Tables

## 4.1 MEGA-1 Pin Assignments

| Component | Pin(s) | Description |
| --- | --- | --- |
| LCD1602 | 12, 11, 5, 4, 3, 2 | RS, E, D4-D7 |
| 4x4 Keypad | 39, 41, 43, 45 (rows) 47, 49, 51, 53 (cols) | Matrix keypad |
| Active Buzzer | 22 | Audio confirmation |
| Passive Buzzer | 13 | Alert tones |
| Push Button | 18 | System start |
| Ultrasonic Sensor | 6 (Trig), 7 (Echo) | Altitude measurement |
| L293D Motor Driver | 28 (IN1), 29 (IN2), 30 (EN) | Fan/propeller control |
| Landing Gear Servo | 27 | Gear deployment |
| MPU6050 | SDA (20), SCL (21) | Attitude sensor |
| 7-Segment Display | 8 (Data), 10 (Clock), 9 (Latch) 23-26 (Digits) | Passcode display |
| IR Receiver | 15 | Remote control |
| I2C Communication | SDA (20), SCL (21) | To MEGA-2 |

## 4.2 MEGA-2 Pin Assignments

| Component | Pin(s) | Description |
| --- | --- | --- |
| LCD1602 | 12, 11, 5, 4, 3, 2 | RS, E, D4-D7 |
| DHT11 | 22 | Temperature/humidity |
| PIR Sensor | 18 | Motion detection |
| Ultrasonic Sensor | 24 (Trig), 26 (Echo) | Object detection |
| Photoresistor | A0 | Light level (night mode) |
| RFID RC522 | 53 (SS), 49 (RST), 52 (SCK) 51 (MOSI), 50 (MISO) | Aircraft detection |
| L293D Motor (Fan) | 8 (IN1), 9 (IN2), 10 (EN) | Cooling fan control |
| Gate Servo | 44 | Gate control |
| Stepper Motor | 31, 33, 35, 37 | Radar simulation |
| Rotary Encoder | 45 (CLK), 43 (DT), 41 (SW) | Airspeed control |
| Passive Buzzer | 7 | Alert tones |
| Yellow LEDs | 28, 30, 32, 34 | Status indicators |
| Red LEDs | 36, 38, 40, 42 | Night mode indicators |
| I2C Communication | SDA (20), SCL (21) | From MEGA-1 |

# 5. Component List

| Component | Quantity | Purpose |
| --- | --- | --- |
| Arduino MEGA 2560 | 2 | Main microcontrollers |
| LCD1602 Display | 2 | User interface |
| 4x4 Membrane Keypad | 1 | Passcode entry |
| DHT11 Sensor | 1 | Environmental monitoring |
| HC-SR04 Ultrasonic | 2 | Distance measurement |
| MPU6050 IMU | 1 | Attitude tracking |
| RFID RC522 Module | 1 | Aircraft identification |
| PIR Motion Sensor | 1 | Motion detection |
| Photoresistor | 1 | Light level sensing |
| L293D Motor Driver | 2 | DC motor control |
| Servo Motor | 2 | Landing gear & gate |
| 28BYJ-48 Stepper | 1 | Radar simulation |
| ULN2003 Driver | 1 | Stepper control |
| Rotary Encoder | 1 | Airspeed adjustment |
| Active Buzzer | 1 | Confirmation beeps |
| Passive Buzzer | 2 | Alert tones |
| 7-Segment Display | 1 | Passcode display |
| 74HC595 Shift Reg | 1 | 7-segment driver |
| IR Receiver | 1 | Remote control |
| Push Button | 1 | System activation |
| LEDs (Yellow) | 4 | Status indicators |
| LEDs (Red) | 4 | Night mode indicators |
| Resistors (220Ω) | 8 | LED current limiting |
| Resistor (10kΩ) | 1 | Photoresistor divider |
| Breadboards | 2 | Circuit assembly |
| Jumper Wires | Various | Connections |

# 6. Code Summary

## 6.1 Key Features

MEGA-1 Code Features:

- State machine with 6 states (passcode entry, verification, temperature setting, running)

- MPU6050 calibration and filtering for accurate attitude readings

- Exponential smoothing for ultrasonic sensor noise reduction

- 7-second minimum landing gear deployment time

- Altitude-based warnings (20cm, 15cm, 6cm thresholds)

- Non-blocking buzzer patterns for pitch/roll/stall warnings

- I2C master transmission of temperature preference

MEGA-2 Code Features:

- I2C slave reception with interrupt handler

- RFID aircraft conflict detection with rotary encoder resolution

- Continuous stepper motor rotation for radar simulation

- Multi-sensor decision logic (DHT11, PIR, ultrasonic, photoresistor)

- Gate and fan automation based on object detection

- Night mode with automatic temperature adjustment to 22°C

- JSON-formatted serial output for monitoring

## 6.2 Critical Functions

MEGA-1:

• deployLandingGear() / retractLandingGear() - Servo control with timing

• checkLandingAltitude() - Multi-threshold altitude monitoring

• updateOrientation() - MPU6050 data acquisition and filtering

• handlePitchBuzzer() / handleRollBuzzer() - Attitude-based warnings

MEGA-2:
• checkRFIDTag() - Aircraft detection and conflict initiation
• handlePlaneConflict() - Conflict mode activation with alarm
• checkRotaryEncoder() - Airspeed adjustment during conflicts
• controlStepperContinuous() - Timed radar scanning rotation

# 7. Testing and Results

## 7.1 System Integration Test

A complete system test verified all subsystems:

- Passcode authentication: Successfully matched and verified 4-digit codes

- I2C communication: Temperature data transmitted reliably from MEGA-1 to MEGA-2

- Altitude monitoring: Landing gear deployed at 15cm, retracted after 7 seconds above 25cm

- MPU6050 attitude tracking: Accurate pitch/roll readings with appropriate warnings

- RFID conflict detection: Successfully triggered conflict mode and airspeed adjustment

- Rotary encoder: Smooth airspeed control from 140 to 100 knots resolved conflict

- Environmental monitoring: DHT11 readings displayed correctly, fan activated above threshold

- Night mode: Photoresistor correctly detected low light and adjusted temperature

- Stepper motor: Continuous rotation provided smooth radar scanning simulation

- Gate control: Servo opened/closed based on ultrasonic object detection

## 7.2 Sensor Accuracy

- Ultrasonic sensors: ±1cm accuracy, 200ms update rate

- MPU6050: ±3° accuracy after calibration

- DHT11: ±2°C temperature, ±5% humidity

- RFID: 100% read success rate within 5cm range

- PIR: 3-5m detection range with 1-2s delay

# 8. Conclusion

This project successfully demonstrates a sophisticated multi-sensor embedded system with dual-microcontroller architecture. The implementation integrates 15+ sensors and actuators, showcasing practical applications of I2C communication, interrupt-driven programming, state machines, and real-time control logic.

Key achievements include:

- Seamless I2C communication between flight computer and ground station

- Advanced RFID-based conflict detection with interactive resolution

- Precise attitude monitoring using 6-axis IMU with filtering

- Automated landing gear deployment with safety timers

- Environmental adaptation through night mode

- Professional code structure with modularity and error handling

The system demonstrates real-world concepts applicable to the aviation industry. Future enhancements could include wireless communication, additional sensor fusion, and integration with flight simulation software.

# 9. Fritzing

Missing components:

| Mega 1 | 4x4 Keypad L293D Motor Driver Landing Gear Servo MPU6050 IR Receiver |
| --- | --- |
| Mega 2 | DHT11 PIR Sensor RFID RC522 L293D Motor (Fan) Gate Servo Stepper Motor |

# Appendix: Source Code

Note: Complete source code for both MEGA-1 and MEGA-2 is included in separate .ino files submitted with this documentation. The code can also be found at https://github.com/tishap27/MEGA-Controller